Single Cycle CPU Report

Seif ElAnsary

Nadine ElGarem

Schematic

The schematic that I created for the full RISV32I single cycle CPU is:

As you can see, we chose to abstract the branching control in a separate branching/jumping control unit. This simplifies the schematic a bit. An extra AluSrc mux was added near to Read data 1 in order to accommodate AUIPC. The data memory now also handles zero/sign extending and the MemtoReg mux was increased in size in order to accommodate, PC + 4, ALU result, LUI and data memory output. ALU control unit was enhanced to support all the new instructions and the Branch mux was expanded to accommodate JALR.

Implementation

We successfully implanted all 37 instructions successfully as you will see in the testing below and there exists no assumptions about any instructions. Each and every bug was swiftly found and dealt with during the exhaustive training we did.

Test Waveforms

- Add
0x00500093 #addi x1, x0, 5  x1 = 5

0x00A00113 #addi x2, x0, 10      # x2 = 10

0x002081B3 #add  x3, x1, x2      # x3 = 15

- Addi

0x00500093 #addi x1, x0, 5       # x1 = 5

0x00A08113 #addi x2, x1, 10      # x2 = 15

- And

0x00600093 #addi x1, x0, 6       # 0110

0x00300113 #addi x2, x0, 3       # 0011

0x0020F1B3 #and  x3, x1, x2      # 0010 = 2

- Andi

0x00600093 #addi x1, x0, 6       # 0110

0x0030F113 #andi x2, x1, 3       # x2 = 2

- AUIPC

0x00001097 #auipc x1, 0x00001     # x1 = PC + 0x1000(4096)

00001097   # auipc x1, 0x00001

- BEQ

0x00500093   # addi x1, x0, 5

0x00500113   # addi x2, x0, 5

0x00208463   # beq x1, x2, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- BGE

0x00500093   # addi x1, x0, 5

0x00400113   # addi x2, x0, 4

0x0020D463   # bge x1, x2, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- BGEU

0xFFF00093   # addi x1, x0, -1

0x00100113   # addi x2, x0, 1

0x0020F463   # bgeu x1, x2, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- BLT

# test_blt.s

0x00200093   # addi x1, x0, 2

0x00400113   # addi x2, x0, 4

0x0020C463   # blt x1, x2, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- BLTU

0x000010B7   # lui x1, 0x00001

0xFFF00113   # addi x2, x0, -1

0x0020E463  # bltu x1, x2, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- BNE

0x00500093   # addi x1, x0, 5

0x00300113   # addi x2, x0, 3

0x00209463   # bne x1, x2, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- JAL

0x008000EF   # jal x1, 8

0x00000513   # addi x10, x0, 0

0x00100513   # addi x10, x0, 1

- JALR

0x00C00093   # addi x1, x0, 12

0x00008167   # jalr x2, x1, 0

0x00000513   # addi x10, x0, 0 (skipped)

0x00100513   # addi x10, x0, 1 (target)

- LB

0x00400103   # lb x2, 4(x0)

- LBU

0x00404103   # lbu x2, 4(x0)

- LH

0x00401103   # lh x2, 4(x0)

- LHU

0x00405103   # lhu x2, 4(x0)

- LUI

0x123450B7   # lui x1, 0x12345

- LW

0x00402103   # lw x2, 4(x0)

- OR

0x00400093   # addi x1, x0, 4

0x00200113   # addi x2, x0, 2

0x0020E1B3   # or x3, x1, x2

- ORI

0x00400093   # addi x1, x0, 4

0x0020E113   # ori x2, x1, 2

- SB

0x06100093   # addi x1, x0, 97

0x00100223   # sb x1, 4(x0)

- SH

0x4D200093   # addi x1, x0, 1234 (04D2 in hex)

0x00101223   # sh x1, 4(x0)

- SLL

0x00100093   # addi x1, x0, 1

0x00300113   # addi x2, x0, 3

0x002091B3   # sll x3, x1, x2

- SLLI

0x00100093   # addi x1, x0, 1

0x00309113   # slli x2, x1, 3

- SLT

0xFFF00093    # addi x1, x0, -1

0x00A00113   # addi x2, x0, 10

0x0020A1B3   # slt x3, x1, x2

- SLTI

0xFFF00093    # addi x1, x0, -1

0x0050A113   # slti x2, x1, 5

- SLTIU

0xFFF00093    # addi x1, x0, -1

0x0010b113   # sltiu x2, x1, 1

- SLTU

0xFFF00093    # addi x1, x0, -1

0x00100113   # addi x2, x0, 1

0x001131b3   # sltu x3, x2, x1

- SRA

0xFF800093   # addi x1, x0, -8

0x00100113   # addi x2, x0, 1

0x4020D1B3   # sra x3, x1, x2

- SRAI

0xFF800093   # addi x1, x0, -8

0x4010D113   # srai x2, x1, 1

- SRL

0x01000093   # addi x1, x0, 16

0x00200113   # addi x2, x0, 2

0x0020D1B3   # srl x3, x1, x2

- SRLI

0x01000093   # addi x1, x0, 16

0x0020D113   # srli x2, x1, 2

- SUB

0x00F00093   # addi x1, x0, 15

0x00500113   # addi x2, x0, 5

0x405101B3   # sub x3, x1, x2

- SW

0xfeedd0b7	#lui x1, 0xFEEDD

0xABCD0093   # addi x1, x0, 0xABC

0x00102223   # sw x1, 4(x0)

- XOR

0x00C00093   # addi x1, x0, 12

0x00A00113   # addi x2, x0, 10

0x0020C1B3   # xor x3, x1, x2 (1100 ^ 1010) = 0110 = 6

- XORI

0x00C00093   # addi x1, x0, 12

0x00A0C113   # xori x2, x1, 10 = 6